SimCPM - a CP/M simulator for the Amiga

by Jim Cathey and Charlie Gibbs

incorporating modifications by Willi Kusche

Version 2.3 - January 9, 1989

This is a FREELY-DISTRIBUTABLE program.  Spread it around all

you like, just don't sell it, and please keep the entire package

(including this file) intact.  If you find any bugs or make any

changes, please let me know - I'd like to keep my copy up to date.

This program is based on an article by Jim Cathey which ran in

Dr. Dobb's Journal from January to March 1986.  The original program

emulated an 8080, including CP/M BIOS and BDOS calls, on a CP/M-68K

system.  I have modified the operating system interface to run under

AmigaDOS (from the CLI only, which shouldn't bother most CP/M-ers :-).

Willi Kusche maintains that the proper term is "emulator", rather

than "simulator."  Although I agree, the original article called it a

simulator, and the name seems to have stuck.

*** NEW FOR VERSION 2.0 ***  Due to popular demand, I have added

support for the entire Z-80 instruction set (including undocumented

opcodes).

WARNING!  This program uses the MOVE from SR instruction to set up

the simulated Z-80 flags.  Therefore, it will not run on the 68010 or

higher processors.  I didn't want to use the Exec's GetCC() call because

it would be too slow.  As a work-around I have set up a EQUate in

options.i which you can change to indicate that you are running on a

68010, 68020, etc.  Re-assembling with x680x0 equated to 1 will cause

all MOVE from SR instructions to be replaced by the corresponding MOVE

from CCR instructions (actually faked with a DC.W in case your assembler

doesn't support 68010 opcodes).  I have assembled a version with this

flag set, and included it in the package; it is named SimCPM010.

It should be possible to check whether x680x0 is equated to 2 and

generate code optimized for a 68020 (as if it wouldn't be fast enough

as is :-).

SimCPM only looks at the first 12 bytes of the FCB (drive code,

file name, and file type) when opening a file.  The remainder of the

FCB is neither altered nor checked, except for BDOS calls 17 (search

for first file) and 18 (search for next file), which insert extent size

and count information only.

If a reference is made to any drive other than A:, SimCPM will

insert the string CPMx: ahead of any file name it uses.  For instance,

if a program tries to open MYFILE.DAT on drive B:, SimCPM will try to

open CPMB:MYFILE.DAT.  You can ASSIGN these simulated drives anywhere

you want.  Drive A: will always go to the current directory, unless the

user number is not zero.  If you specify any user number other than

zero, it will be included, e.g. if the above example were opened under

user 1, SimCPM will look for CPMB01:MYFILE.DAT.  If the file were on

drive A under user 1, SimCPM will look for CPMA01:MYFILE.DAT.

The only built-in commands supported are USER and SAVE.  All

others, such as DIR or TYPE, can easily be done using AmigaDOS

equivalents.  You might want to RUN SimCPM so that the original

CLI window will be available for this.

The EI and DI instructions are currently ignored.

The IN and OUT instructions are ignored except for ports 14 and

15.  Access to the Amiga's serial port is provided through a simulated

Intel 8251 USART, which is currently set up to run at 1200 bps.  Data

is read from or written to port 14, and status input is simulated on

port 15.  All output to port 15 (command) is ignored.  Note that CPU

overhead may not permit an effective throughput of 1200 bps.  (Yup,

you guessed it, it emulates the modem port on my old IMSAI.)

The following BDOS calls are not supported:

3 - reader input

4 - punch output

30 - set file attributes

all above 36

The only BIOS calls supported are those for program termination

and console and list I/O.  (Low-level disk access could get scary.)

If a program attempts to use an unsupported BIOS or BDOS call, an

appropriate error message will be displayed, along with a register dump,

and the program will be terminated.

Basic H19 terminal emulation (similar to a VT52) has been

provided.  Escape sequences for cursor positioning, cursor to home,

screen clearing, and insert and delete modes are translated to the

corresponding Amiga sequences.  The program MANDOG.COM, which I

wrote for my original CP/M (H19) system and copied here unchanged,

demonstrates the cursor addressing capabilities of SimCPM.  Any escape

sequences that SimCPM can't recognize are passed through unchanged.

You can disable all escape sequence translation by changing the equated

value "h19" (in options.i) to from 1 to 0 and re-assembling.

To speed up console output for programs which write one character

at a time, I have included a buffered output option.  If you include

the -b switch on the command-line parameter, all console output will be

held until one of the following events occurs:

- a carriage return, line feed, or bell character is written

- the buffer (currently 2K) becomes full

- a BIOS or BDOS call other than console output is made.

It should be possible to use buffered output in just about any program.

Note that if a program uses the "console output string" function (BDOS

call 9), the entire string is written in a single operation even when

buffering is not requested.

If you include the -t switch on the command line, instruction

and/or BIOS/BDOS tracing is available.  Simulations run somewhat slower

with this flag set, even if tracing is not currently active.  Tracing

is done using 8080 mnemonics by default.  If you want Z-80 mnemonics,

use -z instead of -t.  If both -z and -t are specified, -t will be

ignored.

If the trace switch has been specified, SimCPM will prompt you

for starting and ending addresses for tracing, and whether you want

BIOS/BDOS calls traced.  Simulation starts with tracing disabled.  When

SimCPM executes an instruction at the starting address, tracing is

turned on.  All Z-80 registers and the top four stack entries, as well

as the current instruction, are displayed for each instruction until

the ending address is reached.  Tracing is then disabled until the

start address is again encountered.  After displaying 8 instructions,

execution is suspended; enter S to stop tracing and continue executing,

Q to quit the program, C to change the starting and ending addresses

for tracing, or any other key to trace the next 8 instructions.

If you have requested BIOS/BDOS call tracing, the message

BIOS CALL nn (hex) AT aaaa

or

BDOS CALL nn (hex) AT aaaa

will be displayed each time a BIOS or BDOS call is made.  The BDOS

call number (contents of register C in hex) will replace "nn" for BDOS

calls, while BIOS calls will set "nn" to a sequential number starting

at 01 for program termination, 02 for console status, etc.  The address

where the call is made will be taken from the top of the stack and

displayed in place of "aaaa".

List output is sent to PRT:RAW.  This ensures that no newline

translation will take place.  The printer is not opened until the first

attempt is made to access it.  If the printer cannot be opened for any

reason, the program will be terminated.  At program termination the

printer will be closed if it was used.

If SimCPM is brought up with no command-line parameters, it will

present the standard CP/M A> prompt.  At this point you can run CP/M

.COM files just like on a normal CP/M system.  The command line will

be scanned for up to two FCBs and the tail will be passed to the

program.  When the program terminates, the A> prompt will be

re-displayed.  To return to AmigaDOS, enter a control-C (CP/M warm

boot) at the A> prompt.

If command-line parameters are given, they will be passed to

SimCPM as if they had been entered at the A> prompt.  When the program

terminates, SimCPM will automatically return to AmigaDOS.  This allows

you to run CP/M programs from an EXECUTE file.

Here are some sample invocations of SimCPM:

simcpm

Loads the simulator, which presents the A> prompt and

waits for CP/M commands.  Type control-C at the prompt

to exit.

simcpm -b

Same as above, except console output buffering is used.

simcpm myprog

Loads the simulator, which loads and runs the CP/M

program MYPROG.COM.  When MYPROG terminates, SimCPM

returns to AmigaDOS.

simcpm -t myprog

Same as above, except that before MYPROG.COM is run,

you will be prompted for trace parameters.

simcpm -z myprog

Same as above, except that Z-80 mnemonics will be used

for tracing, rather than 8080 mnemonics.

simcpm mbasic basprog

Loads the simulator, which loads Microsoft BASIC-80 (if

you have it), which in turn loads and runs BASPROG.BAS.

When BASPROG issues a SYSTEM command, SimCPM returns

to AmigaDOS.

simcpm -b mbasic basprog

Same as above, but with console output buffering.  Since

MBASIC does all console I/O one character at a time, the

buffered option can speed it up considerably.

I have included two CP/M programs for demonstration purposes.

TESTPROG.COM is a simple "hello, world" program (try it using the

trace option), while MANDOG.COM demonstrates cursor addressing for

an emulated H19 terminal.

SimCPM consists of two source modules, SIMCPM1.ASM and

SIMCPM2.ASM.  No include files are needed, although amiga.lib is

required for linking.  The makefile is set up to use my own assembler,

A68k, and BLink - it should be relatively easy to adapt it to whatever

assembler and linker you are using.

Special thanks to:

Jim Cathey for his article in Dr. Dobb's Journal -

it provided the starting point for this program.

Willi Kusche (for new ideas and corrections of my Z-80 code)

SYSOP

KMMM Pascal BBS

(609) 232-0773

24 hours a day

7 days a week

300/1200/2400/9600 bps

9600 is USRobotics HST protocol

Passing a simple IQ test required for full access

Please address any kudos, flames, etc. to

Charlie Gibbs

2121 Rindall Avenue

Port Coquitlam, B.C.

Canada

V3C 1T9

or to Larry Phillips or Jeff Lydiatt on CompuServe or Usenet.

(I don't have the time or money to live there myself.)